# Fabrication notes

# hw_DAA/BASIC-101010

September 24, 2024

# 1. PCB construction

- Number of layers: 16 layers.

- Number of flex layers: 6 (3 core structures joint with polyamide bondply).

- A strain relief shall be added over the flex and rigid intersection.

- Surface finish: Immersion silver.

- PCB Material requirement:

  - High TG material, greater than 180ºC.

  - RoHS compliant.

  - REACH compliant.

- Dielectric material: must comply with  IPC-4101-126 standar.

- Soldermask: on both sides of the board, color green.

- All vias in pads shall be plugged with solder mask type V/VI.

- Layer stack shall comply with the IPC-6012C.

- Layout clearance shall comply with the IPC-2221B.

- PCB shall comply with the IPC 6011 Class 3/A.

# 2. Gerber description and stackup

# The layer structure is defined below:

- Layer Top 			→  	*.cmp

- Layer 2 			→  	*.l2

- Layer 3 			→ 	*.l3

- Layer 4 			→ 	*.l4

- Layer 5			→	*.l5

- Layer 6			→ 	*.l6

- Layer 7			→ 	*.l7

- Layer 8			→ 	*.l8

- Layer 9 			→ 	*.l9

- Layer 10			→	*.l10

- Layer 11			→ 	*.l11

- Layer 12			→ 	*.l12

- Layer 13			→ 	*.l13

- Layer 14			→	*.l14

- Layer 15			→ 	*.l15

- Bottom			→	*.sol

- Solder mask top		→ 	*.stc

- Solder mask bottom	→	*.sts

- Dimensions			→	*.dim

- Panel				→	*.gml

- Milling			→	*.gko

- Flex				→	*.flx

- Cream top			→	*.tcr

- Cream bottom		→	*.bcr

- Silkscreen top		→	*.tsilk

- Silkscreen bottom		→	*.bsilk

- Drills				→	*.drd

The PCB stackup is described in the next image:

# 3. Drills

Number of drills: 3241

Drills used:

| Code | Size | Used |
| --- | --- | --- |
| T01 | 0.0059inch | 2874 |
| T02 | 0.0079inch | 61 |
| T03 | 0.0098inch | 17 |
| T04 | 0.0197inch | 154 |
| T05 | 0.0291inch | 32 |
| T06 | 0.0315inch | 68 |
| T07 | 0.0394inch | 4 |
| T08 | 0.0591inch | 1 |
| T09 | 0.0669inch | 4 |
| T10 | 0.1181inch | 4 |
| T11 | 0.1299inch | 4 |
| T12 | 0.1575inch | 12 |
| T13 | 0.2205inch | 6 |

# 5. Silkscreen

The PCB shall have silkscreen in the layers top and bottom.

The gerber data for silkscreen are the *.tsilk and *.bsilk.

Do not include manufacturing logos in the PCB.

The silkscreen color shall be white.

PCB manufacturers shall add a traceability number in the products. This number shall be the PCB batch and a week-year number.

# 6. Additional notes

The PCB manufacturing shall comply with the IPC-1602 standard to ensure a correct handling, protection and storage.

Two additional documents are added for the manufacturer:

- hw_DAA_Rigiflex_drawing.pdf: Describes placement and dimensions of the rigiflex segments.

- hw_DAA_Drill_and_Countersunk_drawing.pdf: Describes the placement and size of the countersunk drills.